# AstroJet SP2

# Service Guide

Part Number: 22834750-EN-E

Rev. 1.0

5/2025

Contents

Section 1: Introduction	6

Intended Audience	6

General Safety Precautions	6

Workplace and Work Methods	6

Clothing	6

Protective Clothing	6

Protective Equipment	7

Safety When Working with Electricity	7

Qualifications of Personnel	7

Maintenance Precautions	7

Additional Precautions for Devices with Exposed Energized Parts	7

Tools	7

If an Accident Occurs	8

Section 2: Getting Acquainted	9

AstroJet SP2 Printer – Front View / Operator Side	9

AstroJet SP2 Printer – Exit View	11

AstroJet SP2 Printer – Rear View (non-operator side)	12

AstroJet SP2 Printer – Ink Door View	13

AstroJet SP2 Printer – Control Panel / Touchscreen	13

AstroJet SP2 Feeder – Exit View	14

AstroJet SP2 Feeder – Entrance View	15

Section 3: Troubleshooting	16

Power Problems	16

Communication Problems	16

Service Tray (Sled) Problems	17

Feeding Problems	17

Toolbox System Status Messages	18

Jams in Printer	20

Removing Jammed Media	20

Misfeeds	20

Section 4: Functional Operation	21

Advanced Service Settings	21

System Settings	21

Events	21

Pzone Servo	21

Lifter Servo	21

Feeder Servo	21

BnB Commands	22

Printhead Movement	23

Section 5: Adjustments	24

Paper Path Drive Motor Belt Tension Adjustment	24

Media Thickness Lift Belt Tension Adjustment	26

Printer Speed Control Setup	27

Adjusting Printer Safety Switch Position (Clamshell)	28

Adjust Media Thickness Home Sensor Height	29

Section 6: Printer Disassembly and Assembly Procedures	30

Printer Basic Disassembly	30

Remove Printer Covers	30

Printer Unit Replacements	32

Replace 24V DC Power Supply	32

Replace Speed Control	32

Replace Aerosol Fans	33

Replace Aerosol Fans Filter	33

Replace Transport Rollers, Belts, and Bearings	34

Replace the Main Motor	37

Replace Media Exit Sensor	38

Replace the Ink Door Switch.	39

Replace the Media Path Encoder Assembly	40

Replace the Media Thickness Motor, Belt and Sensor	41

Replace the Main Power Supply	42

Replace the Fast Internet Switch	42

Replace the Paper Path Drive Motor	44

Replace the Clamshell Switch	44

Replace the Center Plate Assembly.	45

Replace the Registration Table Assembly (Belts Rollers and Bearings)	47

Clamshell Component Replacement Procedures	48

Replace Service Tray (Sled)	48

Replace Print Engine (BnB)	50

Installing a New Print Engine	55

Replace Service Tray (Sled) Drive Mechanism	61

Replace Printer Controller, Touchscreen, or SD Card	63

Replace APC Board (Astro Interface Board)	65

Replace Print Engine Board (MPCA)	67

Section 7: Feeder Component Replacement / Adjustment Procedures	68

Feeder Speed Control Board Setup	68

Remove Feeder Transport Assembly	69

Replace Clutch Assembly	70

Replace Feeder Transport Belts	71

Replace Drive Belt	74

Replace Feed Drive Motor, Encoder	76

Replace Feeder Sensor	78

Replace / Adjust Separator	79

Section 8: Maintenance	81

Maintenance Schedule	82

Replacing Service Tray (Sled)	82

Adjust Feeder Sheet Separator	82

Cleaning	83

Belt Maintenance	83

Print Engine	83

Appendices	84

Appendix A: Printer Specifications	84

Operation	84

Supplies	84

Media	85

Environmental and Physical Requirements	85

Appendix B: Wiring Diagrams	86

Section 1: Introduction

Intended Audience

This service guide is intended for use by qualified service and maintenance personnel. For operation and configuration information, refer to the AstroJet SP2 User Guide. This service guide has assumed that you have read the AstroJet SP2 User Guide in its entirety and that you have it available for reference.

General Safety Precautions

Workplace and Work Methods

Keep the area around the device clean during and after maintenance.

Work in a safety-conscious manner.

Store dismantled device parts in a safe place while maintenance is being performed.

Ensure that tools do not become trip hazards.

Clothing

Warning: Moving parts can draw in clothing and lead to injuries.

If possible, do not wear clothing which could be caught by moving device parts.

Button or roll up shirt sleeves.

Tie or pin up long hair.

Tuck the ends of scarves, ties, and shawls into your clothing, or secure them with non-conductive clips.

Warning: Risk of death from increased flow of current via metal parts which come into contact with the device.

Do not wear clothing with metal parts.

Do not wear jewelry.

Do not wear glasses with a metal frame

Protective Clothing

If any possible danger to your eyes is present, wear protective goggles, especially in the following cases:

When knocking in or knocking out pins and similar parts with a hammer.

When using spring hooks.

When loosening or inserting springs, snap rings, and gripping rings.

When soldering.

When using solvents, cleaning agents, or other chemicals.

Protective Equipment

Warning: Risk of injury in the case of missing or faulty protective equipment.

After performing maintenance work, attach all safety equipment (covers, safety precautions, ground cables, etc.).

Replace faulty parts and those which have become unstable.

Safety When Working with Electricity

Qualifications of Personnel

The following work may only be performed by instructed and trained electricians:

Work on electrical assemblies.

Work on the device while it is open and connected to the power supply

Maintenance Precautions

Locate the Emergency-Stop or Power Switch so that it may be actuated in the case of an emergency

Unplug the device from the electrical outlet before performing the following work:

Removing or installing power supply units.

Working in the immediate vicinity of exposed power supply parts.

Mechanical inspection of power supply parts.

Modifying the device circuits.

Ensure that the device is de-energized.

Check the workplace for possible sources of danger, e.g., moist floors, defective extension cables, faulty protective conduction connections, etc.

Additional Precautions for Devices with Exposed Energized Parts

Give another person the task of remaining near the workplace. This person must be familiar with the location and operation of the Emergency-Stop and Power Switches to switch off the power if danger arises.

Use only one hand while working on electrical circuits when a device is switched on. Hold the other hand behind your back or put it in your jacket pocket. This prevents the electricity from flowing through your body.

Tools

Do not use worn or damaged tools.

Only use tools and testing equipment that is suitable for the respective task.

If an Accident Occurs

Proceed in a cautious and calm manner

Avoid endangering yourself.

Switch the power OFF.

Request medical help (emergency physician) if necessary.

Call for first aid if necessary.

Section 2: Getting Acquainted

AstroJet SP2 Printer – Front View / Operator Side

| 1 | Printer Body – The printer is an A3-width, full-color printing system with an integrated media path. |  |
| --- | --- | --- |
| 2 | Ink Door – The ink door provides access to the ink cartridges. When open, printing is disabled. Upon closing, there is a short delay while ink cartridges are checked for any damage. |  |
| 3 | Clamshell / Print Engine Area (Printzone) |  |
| 4 | Soft-Power Button – Used to power-up or power-down the print engine. To Power-Up – Turn on the main power switch, then press and release the Soft Power Button. The button’s LED will turn blue and the text “Starting up…” will display on the screen. It will take a few minutes for the system to fully initialize. To Power-Down – Press and release the Soft Power Button. The text “Shutting Down…” will display on the screen. If you plan to turn off the Main Power Switch, it is important to wait for the screen to go black and the soft-power button LED to turn off before turning off the Main Power Switch. |  |
| 5 | Touchscreen / User Interface (UI) – Provides access to an extensive range of functionality. |  |
| 6 | Sled Access Cover – Remove to access / remove Shipping Clips, the Service Tray, and the Print Engine BnB |  |
| 7 | Media / TOF Sensor – Moveable sensor for detecting the media entering the printzone from the registration table. Important: Be sure to position the TOF Sensor directly over the media’s path, avoiding cutouts / holes / windows. |  |
| 8 | Media Sensor Reflector – Media (TOF) Sensor must be positioned over this area. |  |
| 9 | Media Guide – Use only when needed to guide media. |  |
| 10 | Media Hold-Down Strap – Used to hold down media as it enters under the Clamshell. |  |
| 11 | Media Transport Belts – To adjust belt position, press “Run Path” then loosen the thumb screws located below this area. Move the belts to desired positions, then secure by tightening the thumb screws. |  |
| 12 | Accutrack Registration Table – Aligns and guides the media into the printzone. |  |
| 13 | Media Registration Rail – Part of the Registration Assembly. Media is registered against this rail. |  |
| 14 | Registration Holder Assembly Knobs – Used to adjust registration roller assembly pressure. |  |
| 15 | Service Access Hole – Set Media Thickness to 12.5 mm to align the screw with the hole. Use T20 Torx Driver to manually drive the Service Tray or BnB in/out of the system when replacement is needed. |  |
| 16 |  | Service Access Handle – Used to open the Top Cover on the Clamshell. Note: The cover is held closed by strong magnets. |

AstroJet SP2 Printer – Exit View

| 1 | Clamshell Release Latches – Must set Media Thickness to 12.5 mm before opening Clamshell. |
| --- | --- |
| 2 | Service Tray (Sled) – Maintains the printhead. Contains a wiper cloth (web) for cleaning the printhead surface. Contains cap to keep printhead nozzles sealed and protected when not in use. |
| 3 | Ink Satellite Exhaust Fans – Used to evacuate ink overspray that is created during printing / purging. |
| 4 | Print Platen and Drip Tray Assembly (Spittoon) – Supports media during printing. Captures excess ink during bled printing and purging processes. Must be removed to inspect and clean periodically. |
| 5 | Exit Conveyor Assembly – Transports media out of the printzone. |
| 6 | Exit Sensor Position – The Exit Sensor is located here. The Exit Sensor is used to confirm proper media movement throughout the printer. If media is not passing over this sensor or there are dark colors / holes in the media passing over the sensor, select “Ignore Exit Sensor”/ Note: Sensor function can be affected by ambient light. Be sure to shield sensor from external light. |
| 7 | Exit Pressure Rollers – Used to help drive the media out of the printzone. Note: Your printer may have versions different than that shown here. Important: Please be sure that the Exit Pressure Roller assemblies are not positioned above the Exit Sensor. |

AstroJet SP2 Printer – Rear View (non-operator side)

| 2 | Media Guide – Used only when needed to help guide media. |
| --- | --- |
| 3 | Registration Roller Assembly – Used to drive media through the registration area and against the registration rail. |
| 4 | Feeder Interface Connection – Connect the Feeder Interface Cable here. |
| 5 | Power Inlet with Power On/Off Switch – Connect the power cord to the power inlet. The power inlet has an on/off switch as well as a drawer that contains two fuses (one in use and one spare). |
| 6 | USB 2.0 Port – Used to connect the printer to a PC using a USB Cable. |
| 7 | Ethernet Port – Used to connect the printer to a network |

AstroJet SP2 Printer – Ink Door View

The printer uses four ink cartridges: Cyan, Magenta, Yellow, and Black (CMYK). Install these cartridges in the slots behind the ink door.

When the Ink Door is open, the printer stops all communications with the Ink Cartridges and the screen displays “Ink Door Open.”

After the Ink Door is closed, it will take up to 30 seconds to read the ink tanks and show the ink levels.

AstroJet SP2 Printer – Control Panel / Touchscreen

The touchscreen on the printer is a control center for media set-up. It can also be used for monitoring printing as well as pausing and restarting jobs plus recovery for any printing errors. Additionally, it provides the ability to store print jobs and allows the user to view and select print jobs for printing.

AstroJet SP2 Feeder – Exit View

| 1 | Exit Roller Assemblies – Used to drive the media from the feeder into the registration area of the printer. |
| --- | --- |
| 2 | Pull-out Roller Pressure Adjustment Knobs – Used to set roller pressure and set the starting height for thicker media. |
| 3 | Side Guide Securing Knobs – Used to secure the position of the Side Guides. |
| 4 | Media Side Guides (Left and Right) – Used to guide and center media in the feeder. |
| 5 | Hopper stacked with media – Shown stacked with #10 Envelopes. |
| 6 | Sheet Separator Adjustment Knob – Use to raise / lower the sheet separator. |
| 7 | Feeder Sensor – Used to detect and measure media length. Note: The Feeder Sensor gets its power from the printer. The Sensor LEDs being ON does NOT indicate that the feeder is powered ON. |
| 8 | Glide Riser Stand – Supports the feeder. Allows for easy repositioning and alignment between the feeder and the printer. |
| 9 | Feeder Position Securing Knob – Used to secure the position of the feeder on the Glide Riser Stand |

AstroJet SP2 Feeder – Entrance View

| # | Name | Description |
| --- | --- | --- |
| 1 | Media Slide Guides | The media side guides hold the media in the correct position for feeding into the printer. |
| 2 | Rear Media Support | Supports the media stack |
| 3 | Feeder Interface Cable | Connect this cable from the back of the feeder to the back of the printer. |
| 4 | Power Inlet with Power On/Off Switch | Connect the power cord to the power inlet. The inlet has a fuse and on/off switch. |
| 5 | Glide Riser Stand | Supports the Feeder Assembly |
| 6 | Jogging Control | Turn OFF/ON Jogging control of the Rear Media Support |

Section 3: Troubleshooting

This section is arranged first by the condition that may occur, and then by possible problems, causes and recommended solutions.

Power Problems

| Condition | Problem | Solution |
| --- | --- | --- |
| Power is ON, nothing happens. | No power to printer. | Check that the power cord is plugged in. Check that the power outlet is live. Check the AC Adapter. |

Communication Problems

| Condition | Problem | Solution |
| --- | --- | --- |
| Cannot open Toolbox | Connection Problems | Network: - Ensure that the printer has an assigned, valid IP address. Ping the printer from your PC to ensure that the printer is accessible. - Refresh your browser (Shift + F5) USB: - Make sure that the printer driver is installed successfully. - Do not use a USB cable that is longer than 10 ft. - If you must use a USB cable longer than 10 ft, it is recommended to insert powered USB hub between the PC and the printer. Refresh your browser (Shift + F5) |

Service Tray (Sled) Problems

| Condition | Problem | Solution |
| --- | --- | --- |
| Service Sled Motor Error | Service Tray (Sled) is skewed. | Eject the Service Tray (Sled) out back and reinsert it, ensuring that it is entering the engine straight, Check the FFC Cable connecting the Sled Motor with the Control Bar. If the motor is still stalled, replace the motor. |
| Color Mixing | Ink migration within the printhead. Common when transporting a primed printer. | Print an alignment page to locate the channel / die affected., Print multiple pages in the affected channel to clear it out. |

Feeding Problems

| Condition | Problem | Solution |
| --- | --- | --- |
|  | No power to printer | Check that the power buttons are ON (Control Panel and main power) and that the power cord is plugged in. |
|  | Double-Feeding | Ensure that media is within specifications. The separator is not adjusted. Rear support is too far back. |
|  | Motor is running, but the clutch is not engaging. | Check that the feeder sensor is not covered. Check that the clutch pin is present. Check the clutch wiring. |
|  | Motor is not running. | Check connections to and from the Speed Control. Ensure that the green LED is ON. Check Encoder feedback from the toolbox. Check the condition of the encoder. |
|  | Media Path Obstruction | Clear the jam and remove the pieces remaining under the printhead. |
|  | Media not loaded properly. | Instruct the operator in the proper loading of media. |
|  | Feeder speed is not matching printer speed. | Feeder ERPI is not correct (Advanced Printer Settings) |

Toolbox System Status Messages

Use the touchscreen or the Toolbox to quickly determine and locate a problem within the printer.

The status indicator shows ERROR in a red box. The printer graphic icon highlights the printer and system affected. The system status information displays the basic problem (in red). Ink levels display the ink status. Control buttons (at the bottom of the screen) allow you to perform frequently used tasks without leaving the screen.

Listed below are some of the messages that may appear in System Status Messages:

| Error Message | Source | Solution |
| --- | --- | --- |
| Paper Motor | Printzone motor Servo Control Loop error. No encoder pulses of error limit reached due to mechanical friction. Reported by APC. | Confirm Error using the Sensor Diagnostic Chart Check the print zone encoder wiring, extension, and connection to an APC (J10) |
| Feed Timeout | Media not detected at Feeder Sensor in time. TOF Sensor detected media without feed. Exit Sensor detected media without feed. Reported by APC. | Confirm Error using Sensor Diagnostic Chart Check if the feeder sensor has two lights on (amber and green) when not interrupted, and green only when interrupted Check the condition of the reflective tape. Clean it of paper dust. Replace the sensor if necessary. |
| Gap Motor | Media Thickness Gap Servo Control Loop error. No encoder pulses of error limit reached due to mechanical friction. Reported by APC. | Confirm error using the Sensor Diagnostic chart. Check the Media Gap Motor and Media Gap Encoder wiring and connections (APC J38 and J34) |
| Gap Sync | Media Gap could not find home position. 1.Motor not turning. Clamshell not moving. 2. GAO Sensor not detecting home position. Reported by APC. | Check motor functionality Check Sensor functionality. Check APC outputs. |
| Feeder Motor | Feeder motor Servo Control Loop error. No encoder pulses of error limit reached due to mechanical friction. Reported by APC. | Confirm error using the Sensor diagnostic chart Inspect the encoder reader wiring and connection Inspect the condition and alignment of the encoder wheel. Ensure there are no scuff marks on the wheel. |
| Feed Overlap | Page length mismatch at Feeder Sensor with following pages longer than the first page. Reported by APC. | Ensure that all media you load is the same size for each job. Ensure that the separator is adjusted properly Ensure that the feeder sensor is functional Ensure that the clutch is functional |
| Print Seq | Excessive TOFs detected, indicates pages not being printed. GPIO signaling may have failed on the print engine. Reported by APC. | Ensure that the separator is adjusted properly Ensure that the feeder sensor is functional Ensure that the clutch is functional |
| Feed Table | Media error between TOF and feeder sensors indicating that media has stalled on the feeder table. Reported by APC. | Confirm error using the sensor diagnostic chart or UI Ensure media is interrupting the sensor as it exits the printer. Clear the face of the sensor of all paper dust, if necessary. Ensure there is no directional light source placed above the sensor. |
| Print Zone | Media error between TOF and Exit Sensor indicating that media has stalled within the print engine. Reported by APC. | Confirm the error using the sensor diagnostic chat or UI Ensure that the media is interrupting the sensor as it exits the printer. Clear the face of the sensor of paper dust, if necessary. Ensure there is no directional light source placed above the sensor. |
| Page Error | Error with the page event handling configuration. | This feature should only be used in conjunction with appropriate support equipment. If not present then this feature should be disable via the Toolbox Service Settings |
| APC Comms | Error communicating to the APC controller board. | The system has the ability to automatically restart the board in most cases. This may take several minutes. If it cannot do this then Clear Error may be required, if enabled. If the status LED on the APC is flashing, then cabling between the APC and UI should be checked. If the status LED is not flashing, then the APC may require replacing. |
| Job Error | The Printhead has reported an error without providing a reason. | If the system cannot be recovered by a Clear Error then the printhead should be reset. |
| PH Feed Timeout | The Printhead has reported a feed timeout during printing. This occurs when the system waits too long for media to arrive at the TOF sensor. | Ensure the media is correctly feeding and the Feed Timeout is correctly set. |
| User Cancel | The Printhead has reported a user cancel request during printing. |  |
| Job Canceled | The Printhead cancelled the job with no specific reason given. | If the system cannot be recovered by a Clear Error then the printhead should be reset. |
| Job Timeout | The Printhead has reported a timeout receiving information during printing. | If the system cannot be recovered by a Clear Error then the printhead should be reset. |
| PH Processing Error | The Printhead has reported an error during printing and cancelled the job. | The Printhead Alerts should be checked on the Diagnostics page to determine what sort of error may be occurring. The Service Technician should be contacted to help resolve the issue. |
| PH Stall | The Printhead has mechanically stalled during printing. | The Printhead Alerts should be checked on the Diagnostics page to determine what sort of error may be occurring. The Service Technician should be contacted to help resolve the issue. If it is a sled stall then it may be resolved by ejecting and correctly reinserting the sled. |
| PH Out of Ink | The Printhead has reported an Out of Ink error during printing. | The ink levels in the supplies should be checked and any empty cartridges replaced. If the problem persists the Service Technician should be contacted to help resolve the issue. |
| Ink Door Open | The Printhead has detected the Ink Door being open during printing. | Ensure the ink door remains closed and checked the wiring and sensor mechanical setup is correct. |
| PE Error | An error has occurred validating the Print Engine. | This may be caused by outdated firmware running on the Print Engine. The Service Technician should be contacted to help resolve the issue. |
| MPCA Error | An error has occurred communicating with the Print Engine. | An error has occurred communicating with the Print Engine controller. The Service Technician should be contacted to help resolve the issue. |
| Ink | An Ink related error has occurred. | The cartridges should be checked to ensure they a authentic cartridges and one of each color is installed. The Service Technician should be contacted to help resolve the issue if valid cartridges are being used. |
| Ink Out | The Printhead Engine has report at least one cartridge is out of ink. | Check the Ink levels and Diagnostics page to determine which cartridge/s are causing the problem. Replace cartridges that are out of ink. The Service Technician should be contacted to help resolve the issue if valid non empty cartridges are being used. |
| Paperpath | A generic issue has occurred with the APC paperpath controller. | The system has the ability to automatically restart the board in most cases. This may take several minutes. If it cannot do this then Clear Error may be required, if enabled. If the status LED on the APC is flashing, then cabling between the APC and UI should be checked. If the status LED is not flashing, then the APC may require replacing. |
| PE Alerts | Print Engine alerts can be displayed on the status screen and will depend on what error has been detected. | The Diagnostics page in the Toolbox should be checked to determine the type of fault that is being reported. Many of these can be cleared using Clear Error but may require some user intervention prior to attempting to Clear Error. If the problem cannot be cleared the Service Technician should be contacted to help resolve the issue. |
| PE Comms Err | Internal network communication with MPCA is interrupted | Make sure there is no internal IP address conflict with another unit in the facility. Check Network switch functionality Make sure MPCA is paired with PE. |

Jams in Printer

If a jam occurs, STOP the printer. Some possible causes of jamming are the following:

  - Damaged media, such as media that is wrinkled or dog-eared (turned down corners).

  - Media that is not still enough may not be suitable.

  - Media caught under the flap of an envelope or stuck to one another.

Removing Jammed Media

If your label media get stuck inside the printer, use the following instructions to remove it:

Caution: When the media is stuck in the print mechanism, do not immediately attempt to pull media out by hand, or printer damage may result. Use the procedure in this section to clear media jams.

- Pause the print job.

- Remove the excess media.

- Access the media path inside the printer.

- Clear the jam.

- When the jam is cleared and the media has been completely advanced or retracted, remove the media from the printer by hand.

- Removing the media by hand should be an easy process. If the media still feels stuck the jam may not have been cleared.

- Caution: Do not attempt to force the media or printer damage may result. Ensure the jam is cleared before removing the media.

- If any excess adhesive has accumulated on the printer during the jam-removal process, you will need to clean it before printing again. Use pre-moistened alcohol cloth wipes to remove adhesive from these areas.

Misfeeds

To correct misfeeds, readjust or replace the sheet separators.

Section 4: Functional Operation

Advanced Service Settings

There are a number of options in the Printer Toolbox that are available only to qualified Service Personnel.

Note: Random updates to the values in Service Settings may occur.

System Settings

From the System Settings Menu, one can set the printer Serial Number and Test Speed.

Important: Invert TOF, Reverse Gap, and Aux Feeder Encoder are functions of legacy versions. Be aware that changing these options may result in error.

Events

When Events is enabled, the printer will broadcast usage data to an assigned URL for diagnostic purposes. Note that an internet connection is required for this function to work properly.

Pzone Servo

Allows for the fine-tuning of Paper Path Motor control.

Lifter Servo

Allows for the fine-tuning of Media Thickness Motor Control (Clamshell up-down).

Feeder Servo

Allows for the fine-tuning of the Feeder Motor control.

BnB Commands

- MFG Mode ON – When MFG Mode is set to ON, the printer will always boot in Manufacturing (MFG) Mode. Restarting the Printhead is required.

- MFG Mode OFF – Turns MFG mode OFF. During the next power-up, the printer will boot in Normal mode. Restarting the Printhead is required.

- MPCA Replace – Pairs a new MPCA with the Print Engine.

- BnB Force Shutdown – Shuts down the Printer Controls without turning down the Print Engine.

- APC OFF/ON – Manually powers up/down the printer I/O board (APC).

- Recalculate BnB IP – Searches local network for IP addresses identical to the internal IP assigned to a Print Engine and Printer Control Board. In the case of a match, it will automatically assign new IP addresses to avoid conflict. Use this option when multiple units are installed in the same location.

- Manufacturing Mode – Used for the testing and integration of a new Print Engine. The function of this is described in the chapter “Replace Print Engine.”

Printhead Movement

The Printhead Movement Menu encompasses a series of commands used to test functions of the Printhead, Lifter Servo Motor, and the Service Sled Motor. Always start with Printhead Move Start and end the test with Printhead Move End.

Section 5: Adjustments

Paper Path Drive Motor Belt Tension Adjustment

- Slide the printer to the edge of table, and loosen up the four screws at the bottom of the printer

- Move the motor as necessary

- Manually spin the tapes so that the belt makes a full revolution before finishing. This allows you to ensure that the tension is correct.

Note: There should be some flexibility in the belt, however, if the belt is too loose, you may have a print quality problem.

Note: If the Paper Path Motor Belt is too loose, it will lead to a banding effect in the printer output.

Media Thickness Lift Belt Tension Adjustment

- Remove the non-operator side cover.

- Loosen the two screws on either side of the assembly.

- Rotate the motor as necessary to loosen the belt.

Note: If the Media Thickness Lift Belt is too loose, it will lead to an inaccurate measurement of printhead / clamshell height

Printer Speed Control Setup

- Ensure that the printer is turned off and that the power cable is disconnected.

- Remove the back and front covers of the printer base.

- Remove the front panel to access the Speed Control.

- Using a non-conductive tool, adjust the potentiometers as shown in the diagram below:

Adjusting Printer Safety Switch Position (Clamshell)

Adjusting Clamshell Switch Position

- Remove the Front Panel.

- Release the following two screws to move the bracket up and down.

- Slide the bracket all the way up

- Close the Clamshell

- Bring the sensor down until either you hear a click, or until you see a signal on the user interface.

Note: Adjust the screws enough to interrupt the switch, but not enough to damage any components.

Note: Setting the switch too low can cause damage to the switch.

Adjust Media Thickness Home Sensor Height

The Media Thickness Mechanism is connected to the motor and cams. As they turn, they raise the mechanism that moves the clamshell

When the flag is triggering the sensor, the printer is in its highest position (12.5 mm). By adjusting a sensor bracket, the vertical movement range of the clamshell can be adjusted.

To adjust the bracket, loosen the two screws (shown above), adjust the bracket, and secure the bracket in place.

Important: Before any repairs or adjustments, it is recommended to mark the position of the sensor bracket. This way, the bracket may be adjusted to the same position.

Note: If the sensor is set too high, it will affect the print quality on very thin media. If the sensor is set too low, you may not be able to print on max-thickness media (12.5 mm).

Section 6: Printer Disassembly and Assembly Procedures

Printer Basic Disassembly

Remove Printer Covers

For most maintenance and repair procedures, the process begins with the following steps:

- Set the Clamshell all the way up

- Turn the printer OFF and unplug the power cord.

- Remove the covers from the printer. The following steps outline how to remove the covers from the machine.

Warning: Do NOT remove the covers from the Exit Section of the machine unless the machine is turned OFF. With the machine on, there is the risk of electrocution!

- Remove the screws connecting the Operator Side Cover and the Non-Operator Side Cover to the machine. See

- Remove the screws connecting the Clamshell Operator Side Cover to the machine.

- The screws connecting the Exit Cover to the machine are only accessible once the Operator and Non-Operator Side Covers have been removed. Unscrew these screws as shown to disconnect the Exit cover:

Printer Unit Replacements

Before performing any Printer Unit Replacement Services, ensure that the Clamshell is in its highest position, the unit is turned OFF, and the power cable is disconnected

Replace 24V DC Power Supply

- Remove the Operator and Non-Operator Side Covers and the Exit Covers of the machine. See Remove Printer Covers on page 31.

- Disconnect all wires connecting to a Power Supply 24V.

- Using a stubby Philips Screwdriver, remove the four screws connecting the 24V DC Power Supply to the machine.

- Install the new 24 DC Power Supply by following the above steps in reverse order.

Replace Speed Control

- Remove the Operator and Non-Operator Side Covers and the Exit Cover of the machine. See Remove Printer Covers on page 31.

- Disconnect each of the labeled cables from their respective ports.

- Remove the screws at the bottom of the assembly connecting the Speed Control to the machine.

- Disconnect each of the labeled cables from their respective ports.

- Install the new Speed Control by following the above steps in reverse order.

Replace Aerosol Fans

- Remove Aerosol Fan Filters. See Replace Aerosol Fans Filter in the module below.

- Open the Clamshell’s Top Cover to access the fan wires.

- Disconnect the Fan Wire Harness at the connector (Quick Connect Mechanism).

- Unfasten the screws holding the Clamshell Exit Cover in place. Remove the Clamshell Exit Cover together with the fans.

- Remove individual fans by unscrewing the remaining two screws.

Replace Aerosol Fans Filter

- Remove the two screws securing the Aerosol Fan Filter to the frame and replace the filter.

Replace Transport Rollers, Belts, and Bearings

- Set the Media Thickness to 12.5 mm to raise the Clamshell to its highest position.

- Remove the Operator and Non-Operator Side Covers. See Remove Printer Covers on page 31.

- Disconnect the Exit Sensor Extension and pull the wire out.

- Unfasten the double hub assembly and disconnect the belts.

- Unfasten the eight screws holding the two-part transport assembly (four on each side).

- Remove the Exit Conveyor Assembly and the Paper Path Assembly together. You will now have access to the transport belts, bearings, and rollers.

- To reassemble, follow the above steps in reverse order.

Note: For reference, the Paperpath Belt Diagram is pictured below:

Replace the Main Motor

- Remove the covers from the printer. See Remove Printer Covers on page 31.

- Unfasten the screws on the bottom of the machine connecting the Main Motor to the machine

- Once these screws are unfastened, remove the Main Motor Belt from its bearings.

- Unfasten the nuts and screw connecting the motor to the frame

- Remove the Main Motor.

- Fasten the new Main Motor to the frame via the same nuts and screws.

- Replace the Main Motor Belt.

- Fasten the screws on the bottom of the machine connecting the Main Motor.

- Replace the covers.

Replace Media Exit Sensor

- Locate the Media Exit Sensor between the belts on the Exit Section of the unit.

- Unscrew the Media Exit Sensor and slide it out with its accompanying bracket.

- Disconnect the sensor via the Quick Disconnect mechanism available.

- Remove the sensor from the bracket.

- Install new sensor.

Note: During installation of the new sensor, make sure that the sensor’s wire harness is routed properly to avoid contact with moving parts.

Replace the Ink Door Switch.

Remove the Clamshell’s Operator Side Cover to get access to the Ink Door Switch. From there, remove the one screw and disconnect the Quick Disconnect mechanism.

Replace the Media Path Encoder Assembly

- Remove the Non-Operator Side Cover

- Disconnect the Encoder Wire Harness Extension.

- Remove the screws connecting the Encoder Bracket to the frame.

- Detach the spring to release all of the encoder tension.

- Loosen the screw from the Encoder Wheel and slide the wheel out.

Note: When assembling the new encoder, ensure that there is enough space between the Encoder Wheel and the Encoder Bracket in order to ensure the wheel’s smooth movement.

- Remove the following three screws and detach the Encoder from the bracket.

- To replace, follow the above steps in the reverse order.

Replace the Media Thickness Motor, Belt and Sensor

Replace Media Thickness Motor or Belt

- Disconnect the Media Thickness Wire Harness from the extension.

- Remove the screws connecting the Media Thickness Motor to the bracket. If you are only replacing a belt, you do not have to fully remove the bracket.

- Remove the Media Thickness Motor and fasten it to the bracket.

- Adjust the belt tension before tightening the screws.

Replace the Media Thickness Motor Sensor

Note: The bracket position is factory adjusted. Mark the current position of the Media Thickness Sensor Bracket before removing it.

- Disconnect the Media Thickness Home Sensor Wire Extension.

- Mark the position of the Sensor Bracket on the frame to make it easier to reinstall in its correct position.

- Remove the screws connecting the Sensor Bracket to the frame.

Once the screws are loose, there should be enough slack to pop the Media Thickness Belt out of position.

- Remove the Media Thickness Belt

- Unfasten the screws connecting the Media Thickness Sensor to the side frame.

- Remove the screws securing the Sensor Assembly to the bracket.

- Replace the new Media Thickness Sensor by following the above steps in reverse order.

Replace the Main Power Supply

- Disconnect the wire connector

- Unfasten the two screws and remove the Main Power Supply.

- Reinstall in reverse order

Replace the Fast Internet Switch

- Remove the Clamshell’s Non-Operator Side Cover.

- Disconnect the three cables connected to the Fast Internet Switch.

- Disconnect the power cable from the fast internet switch.

- Carefully but firmly lift the Fast Internet Switch out of the printer. The Fast Internet Switch is attached to the printer body via hook-and-loop tape.

- Reassemble in reverse order.

Important: Make sure that the power cable does not touch the gears of the Service Tray (Sled) Drive Mechanism. The following picture shows the power cable making contact with the gear. In this case, the Fast Internet Switch should be moved further to the right and / or the power cable should be secured so that it does not contact the gear.

Replace the Paper Path Drive Motor

- Remove the front cover.

- Disconnect the Paper Path Drive Motor from Speed Control (follow the markings A1 A2)

- Pull the printer off the table to access the four screws underneath the printer

- Unfasten these screws and remove the old motor.

- Reassemble in opposite order

Replace the Clamshell Switch

- Remove the Operator Side Cover.

- Disconnect the Clamshell Switch Extension Wire.

- Release the switch from the bracket by removing a single screw.

- Install and connect the new switch.

- Close the clamshell and confirm that the new Clamshell Switch functions as expected. Adjust the Clamshell Switch Bracket vertically as needed.

Replace the Center Plate Assembly.

- Set Media Thickness to 12.5 mm so that the Clamshell is in its highest position.

- Remove the Operator and Non-Operator Side Covers. See Remove Printer Covers on page 31.

- Remove the three screws on the operator side connecting the bottom plate to the frame.

- Remove the three corresponding screws on the non-operator side connecting the bottom plate to the frame.

- Remove the two screws from the bottom of the Clamshell Side Closure.

- Raise the clamshell – The Center Plate Assembly will drop on its own.

- Remove the Center Plate.

- To replace, complete the above procedure in the reverse order.

Replace the Registration Table Assembly (Belts Rollers and Bearings)

- Remove the covers from the machine. See Remove Printer Covers on page 31.

- Remove the screws holding the Media Guides in place and remove the Media Guides.

- Remove the Reflective Tape Plate / Infeed Plate from the Registration Table. You will now be able to disconnect the entirety of the Registration Table from the printer.

- Remove the screws connecting the Registration table to the Printzone. Once these screws are removed, there will be slack on the Drive Belt.

- Disconnect (slide off) the Drive Belt from the Double Bearing.

- With two hands, remove the Registration table. Now you have access to the belts and bearings.

Clamshell Component Replacement Procedures

Note: Before operating on the Clamshell or any of its components, the clamshell may need to be raised to its highest position to allow access to certain components.

Replace Service Tray (Sled)

The Service Tray is a sled assembly within the printer that maintains the printhead’s health. It Is a consumable with its life being displayed on the touchscreen. When it nears the end of its life, users will need to order a new Service Tray and replace the used one using the sled commands on the Toolbox Interface.

- Set the Media Thickness to 12.5 mm to raise the Clamshell to its highest position.

- Open and secure the Clamshell’s Top Cover.

- Open the Sled Access Cover by removing the five screws that secure it.

- From the Toolbox’s Setup Menu, select Sled.

- Select Eject Sled. The Sled will be driven out partially by the Print Engine (BnB)

Important: “Motor Stall” can occur during the Eject Sled process. If this happens, repeat this step. Manually remove the sled using a T20 Torx screwdriver. Insert Sled back in and press Clear Error. Motor will pull Sled back in. Wait a minute for printer to enter IDLE mode before try to Eject Sled again. Do not continue until Sled is ejected without an error.

- Turn a Torx T20 tool clockwise to manually drive the Sled the rest of the way out of the BnB.

Note: You will need to manually pull the sled out once it has disengaged from the drive shaft.

- Install the new Sled. While lightly pushing the Sled into the BnB, drive the Sled into the BnB by turning the Torx T20 tool counterclockwise.

Important: Verify that the Sled is entering the BnB squarely. The trailing edge of the Sled must be parallel with the BnB frame. If not, drive the Sled out and back in again to correct. Drive the Sled into the BnB just enough so that you can replace the Sled Access Cover.

- From the Toolbox’s Setup menu, select Sled.

- Select Install Sled.

  - If installing a NEW Sled, select (check) the New Sled checkbox.

Important: New Sled must only be selected when a NEW Service Tray (Sled) is installed. If New Sled is selected for a used Service Tray, it may cause improper calibration leading to inaccurate life tracking and print quality issues due to inaccurate Sled positioning during head capping and wiping processes.

  - If reinstalling the same Sled that you ejected, the Wiper Life currently displayed should be the correct value for the Sled. In this case, leave the Wiper Life value as is.

  - If reinstalling a Sled from a different printer, enter the Wiper Life value that was displayed at the time the Sled was removed from the other printer.

Replace Print Engine (BnB)

Caution:  Ensure that you are properly grounded before handling the PC Board. Static Electricity can damage the board.

- From the Toolbox, go to View > Setup

- Enter PW (630) in the box located under Service Settings.

- Select BnB Commands

- Select MFG Mode On

- Set the Media Thickness to 12.5 mm to raise the Clamshell to its highest position.

- Once the printhead is in place, power down the machine using the Soft-Power Button.

- Once the Soft-Power Button goes out, turn off the Main Power Switch and unplug the Power Cable.

- Open and secure the Clamshell’s Top Cover.

- Move the magnetic TOF (Top of Form) Sensor so that it is out of the way.

- Remove the five screws fastening the Sled Access Cover in place.

Remove the three screws connecting the Print Engine (BnB) to the Clamshell Frame

- Lock the Service Tray (Sled) in place with Orange Transport Inserts.

Note: This locks the Service Tray (Sled) to the frame of the BnB so that the BnB can be driven out of the Clamshell. This will be done later in the procedure.

Tip: If you have misplaced the Orange Shipping Inserts that came with the printer, you may borrow ones from the new BnB.

- Remove the Ink Cartridges and insert the Orange Ink Compartment Shipping Insert.

- Unlock the lock holding the black ribbon cable in place.

Note: Be sure to follow ESD protocol as static electricity may damage the boards.

- Remove the black ribbon cable by pulling straight down.

- Remove the two white ribbon cables by pulling straight down.

- Secure the three ribbon cables with blue tape so that they do not catch on to the clamshell frame and get damaged as the BnB is removed from the Clamshell.

Note: You may borrow some blue tape from the new BnB.

- Insert the Torx T20 Screwdriver into the opening below to engage the Sled Driveshaft Mechanism. Rotate the screwdriver clockwise to move the BnB until it is completely disengaged from the Sled Driveshaft.

Caution: Take care not to damage the wires and cables as the Service Station enters and exits the unit.

- Using both hands, pull the BnB out all the way.

Installing a New Print Engine

- Unpack the new BnB. Ensure that all blue securing tape is removed, except for the tape that secures the ribbon cables at the top of the BnB

- Insert the new Print Engine (BnB) into the Clamshell until it reaches the Sled Driveshaft. When properly inserted, minimum force is required for this action.

- Insert the Torx T20 screwdriver into the opening below the ink door to engage the Sled Driveshaft. Rotate the screwdriver counterclockwise to drive the BnB in until it reaches a full stop.

Caution: Be careful not to damage the flexible cables as the BnB enters the unit.

- If installed correctly, the BnB frame will lodge into the Clamshell’s frame hardware as shown below:

- Secure the Print Engine by fastening the three screws as shown below:

Caution Ensure that the trailing end of the BnB is sitting flush against the Clamshell frame before inserting and securing screws. Do NOT use the screws to force the BnB to make contact with the Clamshell frame.

- Carefully insert the Flexible Cables into their corresponding sockets on the MPCA Board. Do NOT force cables into sockets as damage may occur.

Check your work to ensure that cables are firmly inserted and not skewed. The black cable must stay in its socket when inserted even before it is locked.

- Remove the orange shipping inserts from the new BnB.

- Insert a NEW set of Ink Cartridges.

Important: You must install a NEW (full) set of ink cartridges. Do not install used cartridges.

- Install the Sled Access Cover and secure using five screws.

- Ensure that the Sensor Wire is properly routed through the cutout. Place the Sensor under the rail on the Sled Access Cover.

- With the ink tanks inserted, power the printer back ON.

- Confirm that the printer is in MFG Mode.

- In the Printer Toolbox, go to View > Setup > Service Settings > BnB Commands > Manufacturing Mode

Following are a series of commands and tests necessary for the correct engine integration. Test results will be visible at the bottom of the screen.

Example:

-1, -1, -1, -1, -1 = Test in Progress

1, 1563,0,0,0 = Test Completed PASS

0, 1254,0,0,0 = Test Completed FAIL

- Execute OBE Bypass > Confirm.

- Execute OBE Done > Confirm.

- Execute Bnb Idle > Confirm.

- Execute Check Sled > Confirm. This will check the function of the Service Tray (Sled) Drive Mechanism.

- Execute Check Printhead > Confirm. This will test the Printhead Positioning Mechanism.

- Execute Mech Init > Confirm. This is an overall engine test.

- Execute Web Spit > Confirm. The printhead should eject ink pattern on the service web.

Note: It is OK if some nozzles are not fired during this test – the important aspect ot see is ink firing.

- If all of the above tests were successful, execute Purge Printhead from the Control Panel.

- Wait for the printer to shut down. At this point, turn the main power switch OFF.

- Install the Purge Tray under the Printhead. Make sure that the tray’s tabs are locked into the Ink Drip Tray. If the tray is not placed properly, ink will spill inside the printer.

- Turn the main power switch back ON. Press the soft power button ON (control panel). In the next 20 minutes, the new Print Engine (BnB) will purge shipping fluid from the system and replace it with ink.

When purging is finished, the printer will go back to IDLE mode.

- Remove the purge tray and dispose of the insert. Do not dispose of the tray.

- Wipe excess ink around the printhead centerplate.

Replace Service Tray (Sled) Drive Mechanism

- Remove the Clamshell’s Non-Operator Side Cover.

- Eject the Service Sled and remove the Service Tray.

- Remove the Center Plate Assembly. You will now have access to the Service Station Sled Motor Assembly.

- To remove the Sled Drive Shaft, loosen the setscrews on the hubs located on both ends of the shaft. Once completed, the shaft is free to remove.

- Unfasten the screws connecting the Service Tray (Sled) Motor to the frame.

- Remove the Service Station Motor.

- Remove the shaft.

- To replace, complete the above steps in reverse order with the new Service Station Sled Motor Assembly.

Replace Printer Controller, Touchscreen, or SD Card

Caution: Save any Stored Jobs to a USB Flash Drive before proceeding.

Caution: Be sure to follow ESD protocol as static electricity may damage the boards.

SD Card

- Open and lock the top cover.

- Remove the four screws securing the control panel to the top cover.

- If you are only replacing the Micro-SD card, it is located on the rPi under the ribbon cable. Gently pull the Micro-SD Card from its port.

Printer Controller (rPi)

- Disconnect the Ethernet Cable, USB Cable, Ribbon Cable, and the Wire Harness.

- Remove the four screws securing the Interface PC Board and the J8 Terminal Bracket to the touchscreen display studs.

Touchscreen

- Disconnect the FFC Cable connecting the Controller to a Touchscreen. Remove the screws connecting the touchscreen to the frame.

- Reconnect all of the appropriate cables to their assigned ports.

- Slide the new touchscreen in ensuring that no cables are getting pinched.

- Replace by repeating the above steps in reverse order.

Replace APC Board (Astro Interface Board)

Caution: Ensure that you are properly grounded before handling the PC Board. Static Electricity can damage the board.

- Turn the printer OFF and unplug the power cable.

- Open and secure the top cover.

- Disconnect the APC Board Power Cord from terminal J3

- Disconnect the remaining wire harnesses from the APC Board terminals.

Note: The wire harnesses and PC Board are labelled for easy reconnection. Use the APC Schematic to check your work before you power the APC board back up.

Caution: Do not damage wires during removal.

- Remove the four screws that attach the PC Boad to the top cover.

- Remove the APC Board.

- Replace by following the above steps in the opposite order

Replace Print Engine Board (MPCA)

Caution: Ensure that you are properly grounded before handling the PC Board. Static Electricity can damage the board.

Replace MPCA

- Turn the printer OFF and unplug the power cable.

- Open and secure the top cover.

- Unplug the MPCA Power Supply Cable from the J9 terminal on the board.

- Disconnect the remaining wire harnesses from the MPCA board terminals. Note that the wire harnesses and the PE Board are labeled for easy reconnection.

- Remove the screws attaching the MPCA to the Top Cover and remove the board.

- To reassemble, follow the above steps in reverse order.

Pair Existing Print Engine (BnB) with the new MPCA

- Power up the printer.

- From the Toolbox, navigate to View > Setup > Service Settings > BnB Commands.

- Execute MPCA Replace > Confirm.

- Execute BnB Force Shutdown. Wait until the printer is shut down to turn the main power switch OFF.

- Wait 30 seconds before turning the main power switch back ON.

- Turn the printer ON.

Section 7: Feeder Component Replacement / Adjustment Procedures

Feeder Speed Control Board Setup

- Remove the Operator Side Panel to access the Speed Control Board.

- Using a non-conductive tool, adjust the potentiometers on the Speed Control Board as shown below.

Remove Feeder Transport Assembly

- Remove the Operator and Non-Operator Side Covers from the Feeder.

- Disconnect the Clutch Wire Harness from the Clutch Ext. Wire Harness at the connector.

- Remove the screws securing the Feed Drive on both sides of the Feeder (1).

- Loosen the screws securing the Exit Roller Rod and the screws securing the Feeder base on both sides of the machine (2).

- Slide the Drive Belt off of the motor. The Feeder Transport Assembly is now free.

- Pull the Feeder Transport Assembly out.

- To reassemble, follow the above steps in reverse order.

Replace Clutch Assembly

- Remove the Feeder Transport Assembly. See Remove Feeder Transport Assembly on page 72.

- Remove the Pulley Cover.

- Remove the screw securing the Clutch Roller Shaft to the assembly frame (Operator Side).

- Push the bearing out to release the Clutch Shaft.

- Pull the Clutch Shaft out just enough to free the clutch.

- Without completely removing the shaft from the assembly, compress the Clutch Spring enough to release the Security Pin. Do not lose the Security Pin.

- Slide the Clutch out.

- Install a new Clutch in the place of the old one.

- Push the Clutch down the shaft to compress the spring.

- Reinstall the Security Pin.

- Release the Clutch, ensuring that the ring covers the pin completely.

- From here, follow steps 4-1 in reverse order to reassemble. Ensure that the Clutch is properly fitted to the frame pin.

Replace Feeder Transport Belts

- Remove the Feeder Transport Assembly. See Remove Feeder Transport Assembly on page 72.

- Remove all screws from the Operator Side of the Feeder Transport Frame.

Note: The screws on the non-operator (clutch) side of the Feeder have Loctite on them to keep them from loosening due to the direction of shaft rotation. The screws on the operator side do NOT have Loctite on them as they will not loosen because of the direction of shaft rotation. Therefore, when performing belt replacement, it is best to remove the screws and frame at operator side.

- Once the frame is removed, you will have access to the belts. Replace the belts.

- Align the belts with the Belt Guides for easier reassembly. Ensure that the Drive Belts remain aligned before reassembly.

- Install the Feeder Transport Frame.

- Connect the Belt Guiding Rollers. Do not tighten the screws all the way, to allow supporting components to fit.

- Install all supporting components.

Installation Tip: Start with the screws that secure the two belt guide plates. Install the screws just far enough to capture the plates. Then, align the entry and exit shafts and install the shoulder bearings and screws for these two shafts. After this, tighten the screws for the belt guide plates. Verify that all belts are properly aligned between belt guides and are as straight as possible. Use a flat tool or a screwdriver fitted between the belt guide [plate and the large roller to pry the roller into position to get the shoulder bearing installed. Repeat this process for the other large roller, then install the remaining screws.

- Tighten all screws.

Replace Drive Belt

- Remove the Feeder Transport Assembly. See Replace Feeder Transport Assembly on page 69.

- Remove all screws from the Clutch Roller and Adjacent Roller starting on the Operator’s Side. There is no need for disassembly.

- Pull the Clutch Roller and the Adjacent Roller out just enough to free the Drive Belt.

- Replace the Drive Belt.

- Connect the Belt-Guiding Rollers. Do not tighten screws all the way to allow for other components to fit.

- From here, reassemble in reverse order.

Replace Feed Drive Motor, Encoder

- Turn the Feeder OFF and disconnect the power.

- Disconnect the motor from the Speed Control Board.

- Disconnect the Encoder Wire from the Encoder Reader.

- Remove the Non-Operator Side Cover from the machine.

- Turn the Feeder sideways.

- Unfasten the screws connecting the Feed Drive Motor to the Feeder Base.

- Disconnect the Feed Drive Belt from its bearing.

- Remove the Motor from the Feeder.

- Remove the Encoder Reader, Encoder Code Wheel, and Bracket from the old motor. There are two spacers behind the bracket – ensure that they are transferred to a new motor.

- Install the Encoder Components to a new Motor. To ensure a centered position within the Encoder Reader, ensure that the Encoder Code Wheel is installed 0.25” (6.0 mm) from the bracket. The Code Wheel must remain clean and undamaged. If there are scuff marks on it, replace it.

- Ensure that the Encoder Reader is installed so that the arrow is on the side away from the motor body, pointing downwards.

- Install the Motor in reverse order.

- Connect the Motor to the Speed Control Board and the Encoder to the Encoder Wire Harness.

- Install the Drive Belt and adjust belt tension.

Replace Feeder Sensor

- Replace the Operator Side Feeder Cover.

- Disconnect the Feeder Sensor Wire Extension.

- Remove the screws connecting the Feeder Sensor to the bracket (two).

- Pull the Sensor Wire out.

- Install the new Feeder Sensor.

- Secure the Feeder Sensor Wire and reconnect it to the Sensor Wire Extension.

Replace / Adjust Separator

- Locate the Separator Mounting Screw and remove it.

- Leave the Sensor Assembly connected to the Separator Assembly.

To Adjust the Separator

- Loosen the (two) screws securing the Separator Wheel in place

- Turn the Separator Wheel just enough to position the unused section of the wheel at the bottom.

To Replace the Separator

- Remove the (two) screws securing the Separator Wheel in place.

- Remove the existing Separator Wheel and install a new one in its place.

- Tighten the screws securing the Separator Wheel to prevent involuntary spinning of the wheel.

- Fasten the Separator Assembly back to the Feeder Body.

Section 8: Maintenance

General, periodic maintenance is needed to keep the printer in good working order. Maintenance can be performed by operators with basic supplies and without special tools. Refer to the User Guide for details.

All other maintenance tasks should only be performed by trained service personnel. These tasks are described in the following chapter.

Note: High volume usage may require more frequent maintenance.

Maintenance Supplies & Equipment:

  - Flashlight

  - Small Telescoping Mirror

  - Hard-bristled toothbrush

  - Tweezers

  - Small, Flathead Screwdriver.

  - Powder-free Nitrile Gloves

  - Protective Clothing and Eyewear

  - Vacuum with Wand

  - Deionized / Distilled Water

  - Super Lube 21030 Synthetic Grease (or equivalent)

  - Loctite 38650 Copper Anti-Seize (or equivalent)

  - Can of compressed air

  - Foam or Lint-free Cotton Swabs

  - Lint-free wipes

  - Disposable shop towels.

  - T20 Torx Driver with 6” or longer shaft

  - #1, #2 Philips Screwdrivers

  - #1, #2 Flat Screwdrivers

  - Set of SAE Allan Wrenches

  - Other common tools

Maintenance Schedule

| Part | Action |
| --- | --- |
| DAILY MAINTENANCE |  |
| Check Ink Levels | Make sure that you have enough ink for the day of printing. |
| Print Platen w/ Spittoon | Wipe ink residue from the platen surface. |
| Media Sensors | Make sure that sensor tape is clean. |
| BI-WEEKLY MAINTENANCE |  |
| Print Platen w/ Spittoon | Check waste ink level. Flush and dry ink tray if needed. |
| Print Zone | Cleaning – Remove media fibers and ink residue. |
| Service Sled | Check remaining Service Sled Life |
| Sensors | Check Exit Sensor. Clean with dump Q-tip if needed. |
| MONTHLY MAINTENANCE |  |
| Transport | Clean transport and feeder belts with soap and water. |
| EVER SIX MONTHS |  |
| Check Firmware Version | Verify latest version and update if needed. |
| Transport | Verify Smooth Operation. Listen to noise indicating damage or wear. |

Warning:  Always power down the printer before connecting or disconnecting any wiring harnesses or cable connections to avoid serious shock or injury.

Caution: Always use appropriate Personal Protection Equipment (PPE)

Caution: Use Electrostatic Discharge (ESD) protection when maintaining equipment.

Caution: Dispose of all maintenance waste in accordance with local regulations.

Replacing Service Tray (Sled)

The Service Tray (Sled) is a consumable with its lifespan available for display on the printer’s touchscreen. When it nears the end of its life, users will need to order a new Service Tray and replace the used one.

To locate the replacement procedure for the Service Tray, see Replace Service Tray (Sled) on page 48.

Adjust Feeder Sheet Separator

To locate the adjustment procedure for the Feeder Sheet Separator, see Replace / Adjust Separator on page 80.

Cleaning

Dust and ink smudges may eventually appear on the printer exterior. Use the following instructions to clean the printer as needed.

- Ensure that the printer is powered OFF. If the printer is on, press the power button for no less than two seconds and then release it. The power button will light red, and the printer will shut down.

- Dampen a soft cloth with water. Then, gently clean the printer exterior with cloth.

Caution: Do not use household cleaners or detergents to clean the printer exterior or printer damage may result.

Belt Maintenance

Belts are consumables that will need to be replaced after extended periods of time and use. Locate the following procedures in this manual for different belt replacement procedures.

To replace the Transport Belts, see Replace Transport Rollers, Belts, and Bearings on page 35.

To replace the Registration Table Belts, see Replace the Registration Table Assembly (Belts Rollers and Bearings) page 48.

To replace the Feeder Transport Belts, See Replace Feeder Transport Belts on page 72.

Print Engine

The Print Engine (BnB) is a consumable that may need to be replaced and reinstalled after extended use.

To locate the replacement procedure for the Print Engine, see Replace Print Engine (BnB) on page 51.

To locate the installation procedure for the Print Engine, see Installing a New Print Engine (BnB) on page 56.

Appendices

Appendix A: Printer Specifications

The technical specifications for the AstroJet SP2 operation, supplies, media, and environment are shown in the following tables:

Operation

| Operation |  |
| --- | --- |
| Print Technology | Thermal Inkjet |
| Ink Type | Pigment-based ink, 4 individual CMYK cartridges. HP TIJ 4.0 Technology. |
| Print Resolution | 1200 dpi x 1200 dpi @ 9 ips (13.5 m/min) 600 dpi x 1200 dpi @ 18 ips (27 m/min) |
| Print Speed | Up to 6,500 Letter/A4 pages per hour Normal Quality: (1200 x 600 dpi) Up to 10,000 #10 envelopes/hour Up to 5,000 Letter pages/hour Best Quality: (1200 x 1200 dpi) Up to 15,000 #10 envelopes/hour Up to 2,500 Letter pages/hour |
| Duty Cycle | 500,000 Letter/A4 sheets per month |
| Feeder Capacity | 16.0” (406 mm) Feed Hopper |
| Print Area | Up t 11.7 W x 30” L with full bleed capacity for media under 11.5” |
| Connectivity | USB 2.0; 802.3 LAN (10/100/100) Ethernet Port |
| Software | Microsoft Windows® 11, 10, 64/32-bit Driver, with built-in color profiles Digital firmware updates via PC Electronic thickness control via printer touchscreen TrueType or PostScript system fonts RIP Software Available (optional) |
| Color Matching | ICC Color Profile support for qualified media. Color management: Windows Driver with Color Controls |

Supplies

| Supplies |  |
| --- | --- |
| Ink Cartridges | CMYK Aqueous Pigment Inks: C:238 ML M: 233 ML Y: 225 ML K: 498 ML ISO Pages: K: 20,000 pages CMY: 16,000 pages |
| Maintenance | Replaceable Wiper Service Tray |

Media

| Media |  |
| --- | --- |
| Width | 3.0” (76 mm) – 14” (355mm) |
| Length | 3.75 (95 mm) – 15.0” (380 mm) |
| Thickness | 0.004” (0.1 mm) – 0.500” (12.5 mm) |

Environmental and Physical Requirements

| Environmental & Physical Requirements |  |
| --- | --- |
| Printer Operating | 15º C to 30º C (59º F to 86º F) 30% to 80% Relative Humidity (non-condensing) |
| Printer Storage | -40º C to 60º C (-40º F to 140º F) |
| Ink | -40º F to 140º F (-40º C to 60º C), 30% to 80% Relative Humidity (non-condensing) |
| Power Requirements | Input: 115-240 VAC 50/60Hz |
| Duty Cycle | 500,000 per month |
| Dimensions | Printer Only” 23” W x 38” L x 22” H With Feeder: 24” W 55” L x 26” H |
| Weight | Printer Only: 145 lbs. With Feeder: 195 lbs. 110 lbs. (50 kg) 145 lbs. printer only 20 lbs. (9.1 Kg) shipping |
| What’s Included in the Boxes | Larger Box: AstroJet SP2 printer and registration table, AC Power Cord, USB Cable, Installation Guide Smaller Box: Media Feeder with Media Guides and media holder, table for media feeder, AC Power Cord. |

## Errors and Warnings

### Toolbox System Status Messages

Use the touchscreen or the Toolbox to quickly determine and locate a problem within the printer.

The status indicator shows ERROR in a red box. The printer graphic icon highlights the printer and system affected. The system status information displays the basic problem (in red). Ink levels display the ink status. Control buttons (at the bottom of the screen) allow you to perform frequently used tasks without leaving the screen.

Listed below are some of the messages that may appear in System Status Messages:

| Error Message | Source | Solution |
| --- | --- | --- |
| Paper Motor | Printzone motor Servo Control Loop error. No encoder pulses of error limit reached due to mechanical friction. Reported by APC. | Confirm Error using the Sensor Diagnostic Chart Check the print zone encoder wiring, extension, and connection to an APC (J10) |
| Feed Timeout | Media not detected at Feeder Sensor in time. TOF Sensor detected media without feed. Exit Sensor detected media without feed. Reported by APC. | Confirm Error using Sensor Diagnostic Chart Check if the feeder sensor has two lights on (amber and green) when not interrupted, and green only when interrupted Check the condition of the reflective tape. Clean it of paper dust. Replace the sensor if necessary. |
| Gap Motor | Media Thickness Gap Servo Control Loop error. No encoder pulses of error limit reached due to mechanical friction. Reported by APC. | Confirm error using the Sensor Diagnostic chart. Check the Media Gap Motor and Media Gap Encoder wiring and connections (APC J38 and J34) |
| Gap Sync | Media Gap could not find home position. 1.Motor not turning. Clamshell not moving. 2. GAO Sensor not detecting home position. Reported by APC. | Check motor functionality Check Sensor functionality. Check APC outputs. |
| Feeder Motor | Feeder motor Servo Control Loop error. No encoder pulses of error limit reached due to mechanical friction. Reported by APC. | Confirm error using the Sensor diagnostic chart Inspect the encoder reader wiring and connection Inspect the condition and alignment of the encoder wheel. Ensure there are no scuff marks on the wheel. |
| Feed Overlap | Page length mismatch at Feeder Sensor with following pages longer than the first page. Reported by APC. | Ensure that all media you load is the same size for each job. Ensure that the separator is adjusted properly Ensure that the feeder sensor is functional Ensure that the clutch is functional |
| Print Seq | Excessive TOFs detected, indicates pages not being printed. GPIO signaling may have failed on the print engine. Reported by APC. | Ensure that the separator is adjusted properly Ensure that the feeder sensor is functional Ensure that the clutch is functional |
| Feed Table | Media error between TOF and feeder sensors indicating that media has stalled on the feeder table. Reported by APC. | Confirm error using the sensor diagnostic chart or UI Ensure media is interrupting the sensor as it exits the printer. Clear the face of the sensor of all paper dust, if necessary. Ensure there is no directional light source placed above the sensor. |
| Print Zone | Media error between TOF and Exit Sensor indicating that media has stalled within the print engine. Reported by APC. | Confirm the error using the sensor diagnostic chat or UI Ensure that the media is interrupting the sensor as it exits the printer. Clear the face of the sensor of paper dust, if necessary. Ensure there is no directional light source placed above the sensor. |
| Page Error | Error with the page event handling configuration. | This feature should only be used in conjunction with appropriate support equipment. If not present then this feature should be disable via the Toolbox Service Settings |
| APC Comms | Error communicating to the APC controller board. | The system has the ability to automatically restart the board in most cases. This may take several minutes. If it cannot do this then Clear Error may be required, if enabled. If the status LED on the APC is flashing, then cabling between the APC and UI should be checked. If the status LED is not flashing, then the APC may require replacing. |
| Job Error | The Printhead has reported an error without providing a reason. | If the system cannot be recovered by a Clear Error then the printhead should be reset. |
| PH Feed Timeout | The Printhead has reported a feed timeout during printing. This occurs when the system waits too long for media to arrive at the TOF sensor. | Ensure the media is correctly feeding and the Feed Timeout is correctly set. |
| User Cancel | The Printhead has reported a user cancel request during printing. |  |
| Job Canceled | The Printhead cancelled the job with no specific reason given. | If the system cannot be recovered by a Clear Error then the printhead should be reset. |
| Job Timeout | The Printhead has reported a timeout receiving information during printing. | If the system cannot be recovered by a Clear Error then the printhead should be reset. |
| PH Processing Error | The Printhead has reported an error during printing and cancelled the job. | The Printhead Alerts should be checked on the Diagnostics page to determine what sort of error may be occurring. The Service Technician should be contacted to help resolve the issue. |
| PH Stall | The Printhead has mechanically stalled during printing. | The Printhead Alerts should be checked on the Diagnostics page to determine what sort of error may be occurring. The Service Technician should be contacted to help resolve the issue. If it is a sled stall then it may be resolved by ejecting and correctly reinserting the sled. |
| PH Out of Ink | The Printhead has reported an Out of Ink error during printing. | The ink levels in the supplies should be checked and any empty cartridges replaced. If the problem persists the Service Technician should be contacted to help resolve the issue. |
| Ink Door Open | The Printhead has detected the Ink Door being open during printing. | Ensure the ink door remains closed and checked the wiring and sensor mechanical setup is correct. |
| PE Error | An error has occurred validating the Print Engine. | This may be caused by outdated firmware running on the Print Engine. The Service Technician should be contacted to help resolve the issue. |
| MPCA Error | An error has occurred communicating with the Print Engine. | An error has occurred communicating with the Print Engine controller. The Service Technician should be contacted to help resolve the issue. |
| Ink | An Ink related error has occurred. | The cartridges should be checked to ensure they a authentic cartridges and one of each color is installed. The Service Technician should be contacted to help resolve the issue if valid cartridges are being used. |
| Ink Out | The Printhead Engine has report at least one cartridge is out of ink. | Check the Ink levels and Diagnostics page to determine which cartridge/s are causing the problem. Replace cartridges that are out of ink. The Service Technician should be contacted to help resolve the issue if valid non empty cartridges are being used. |
| Paperpath | A generic issue has occurred with the APC paperpath controller. | The system has the ability to automatically restart the board in most cases. This may take several minutes. If it cannot do this then Clear Error may be required, if enabled. If the status LED on the APC is flashing, then cabling between the APC and UI should be checked. If the status LED is not flashing, then the APC may require replacing. |
| PE Alerts | Print Engine alerts can be displayed on the status screen and will depend on what error has been detected. | The Diagnostics page in the Toolbox should be checked to determine the type of fault that is being reported. Many of these can be cleared using Clear Error but may require some user intervention prior to attempting to Clear Error. If the problem cannot be cleared the Service Technician should be contacted to help resolve the issue. |
| PE Comms Err | Internal network communication with MPCA is interrupted | Make sure there is no internal IP address conflict with another unit in the facility. Check Network switch functionality Make sure MPCA is paired with PE. |

Appendix B: Wiring Diagrams